Firefighting Drone Swarm

Iteration 0

Group 9

Michael Anastasakis

Tina Cao

Jacob McKee

Damon Ricci

Andrew Tawfik

Dylan Tombs-Koscinski

Drone Model: IF1200

1) Travel time between zones (Damon)

Steps the drone needs to take to travel between zones

- Reach altitude

- Move to given (x2, y2), Starts at (x1, y1) or (0, 0) if at home base

  - Start at 0m/s

  - Accelerates at 3.5m/s^2 (with payload) until speed of 25m/s is reached

  - Should decelerated at a calculated time to stop exactly at destination (x2, y2)

- Return to (0, 0)

- Descend from the altitude to 0

Assumptions

- No drag on the drone object due to wind

- Density of the fluid (air) through which the object is moving is negligible

- The drone does not need to tilt to move in a given direction

- Deacceleration is applied instantly when invoked

- Deacceleration is achieved by the drone object inverting the propellers

- Ascent action of drone comes to a hard stop when horizontal flight height is reached (time to deaccelerate = 0 s)

- descent action of drone comes to a hard landing (v = 0 m/s) when moving to the ground (time to deaccelerate = 0 s)

Kinematics Equations

vf = vi + a*t

vf = a*t

t = vf /a		[1a]	time to accelerate to max velocity at constant acceleration from rest

t = vi /a		[1b]	time to decelerate to rest at constant acceleration from max velocity

vf^2 = vi^2 + 2aΔd

Δd = (vf^2) / 2a	[2a]	Distance covered during acceleration from rest

Δd = (vi^2) / 2a	[2b]	Distance covered during deceleration from max velocity

Δdhorizontal  = SQRT( x2^2 + y2^2)		[3a]

Δdconstant,hor  = Δdhorizontal - Δdaccelerate - Δddeaccelerate	[3b]

Δdconstant,ver  = Δdvertical - Δdaccelerate		[3c]

v = Δd/t

t =  Δd/v		[4] 	Time to move Δd distance at a constant velocity

t =  vmax/a		[5] 	Time to accelerate to max velocity

Ascent (assume hard stop):

t1

to max velocity:		vi = 0, vf = 5 m/s, a1 =	3.27 m/s^2

Δdaccelerate = (vf^2) / 2a1                  [2a]

t1 =  Δdaccelerate/vmax	        [4]

t2

Constant Velocity:		v = 5 m/s, a1 = 0, d = ?

Δdconstant,ver  = Δdvertical - Δdaccelerate		[3c]

t2 =  Δdconstant,ver/vmax  	           [2a,2b,3c,4]

Horizontal To x2, y2:

t3

to max velocity:		vi = 0, vf = 25 m/s, a1 = 3.27 m/s^2

Δdaccelerate = (vf^2) / 2a1                 [2a]

t3 =  Δdaccelerate/vmax	        [4]

t4

Constant Velocity:		v = 25 m/s, a = 0, d = ?

t4 =  Δdconstant/vmax  	 [2a,2b,3a,3b,4]

t5

Constant Velocity to rest:	vi = 25 m/s, vf = 0, a1 = 3.27 m/s^2

Δddeaccelerate = (vf^2) / 2a1   	     [2b]

t5 =  Δddeaccelerate/vmax	        [4]

t6-8

Horizontal Return:

Same as Horizontal travel to x2, y2 but acceleration changes with less load (balls deployed)

a2 = 4.38 m /s^2

t9

Decent (assume hard landing):

to max velocity:		vi = 0, vf = 2.5 m/s, a2 = 4.38 m /s^2

Δdaccelerate = (vf^2) / 2a2     	     [2a]

t9 =  Δdaccelerate/vmax	        [4]

t10

Constant Velocity:		v = 2.5 m/s, a = 0, d = ?

Δdconstant,ver  = Δdvertical - Δdaccelerate		[3c]

t10 =  Δdconstant,ver/vmax  	        [2a,2b,3c,4]

Total Time Travel Between Zones

tnet = t1 + t2 + t3 + t4 + t5 + t6 + t7 + t8 + t9 + t10

2) Time to open/close nozzle doors

N/A

3) Rate of dropping water/foam

3.1) Rate of dropping foam

Drop rate is acceleration due to gravity. Each ball covers a 1.29m radius to contain the fire. Deployment time once in the fire is 3 seconds. [1].

Deployment device weighs 1.7kg, 8kg fully loaded. 8 bombs fully loaded. [2]

Each of the 8 balls is .7875 kg (~ .8L) / 3s = 2.13 L/s rate of foam material

Time to initiate drop (estimate) 0.1 seconds.

Time to fall to fire: depends on the height of the drone.

Deployment time: 3 seconds.

3.2) Rate of flow from nozzle

N/A unless we decide to have different types of drones.

4) Acceleration/deceleration for takeoff/landing (Andrew)

Acceleration Takeoff (with 8x fire extinguishing balls ~ .7875 kg per ball):

aIF1200 = 5 m/s2, mIF1200 = 16.3 kg, mpayload = 8.6 kg

FIF1200 = mIF1200 × aIF1200

FIF1200 = 16.3 kg ×5 m/s2

FIF1200 = 81.5 N

mtotal =mIF1200 + mpayload

mtotal =16.3 kg+ 8.6 kg

mtotal = 24.9 kg

atotal = FIF1200mtotal

atotal = 81.5 N24.9 kg

atotal = 3.27 m/s2

Acceleration Landing (with 0x fire extinguishing balls ~ .7875 kg per ball):

mpayload empty = mpayload, 8 balls - 8× mball

mpayload empty =  8.6 kg- 8× .7875 kg

mpayload empty  =  8.6 kg- 6.3 kg

mpayload empty =  8.6 kg- 6.3 kg

mpayload empty =  2.3 kg

mtotal empty =mIF1200 + mpayload empty

mtotal empty  =16.3 kg+ 2.3 kg

mtotal empty = 18.6 kg

FIF1200 = 81.5 N

atotal empty = FIF1200mtotal empty

atotal empty  = 81.5 N18.6 kg

atotal empty = 4.38 m/s2

References

[1] https://www.passivefirewarehouse.com.au/the-fire-ball-by-ELIDE-FIRE

[2] https://www.uavfordrone.com/product/drone-uav-fire-extinguishing-ball-for-dji-matrice-600-pro/

[3] https://storage.googleapis.com/inspiredflight-com/uploads/IF1200%20Spec%20Sheet%20.pdf

Link for Google Sheet

https://docs.google.com/spreadsheets/d/1Hk4RibSerDurP3eRSJIjgoWIRVVZ9wUEVBlHmG0VIJk/edit?usp=drive_link